Dr Y S R HORTICULTURAL UNIVERSITY

VENKATARAMANNAGUDEM – 534 101

BID DOCUMENT

FOR

CONSTRUCTION OF SCREENHOUSE ALONG WITH IT’S ACCESSORIES ON TURN KEY BASIS

AT

HORTICULTURAL RESEARCH STATION - MAHANANDI

NANDYAL DISTRICT

ANDHRA PRADESH

HORTICULTURAL RESEARCH STATION - MAHANANDI

NANDYAL DISTRICT

ANDHRA PRADSEH

PIN: 518502

Dr. Y S R HORTICULTURAL UNIVERSITY

HORTICULTURAL RESEARCH STATION-MAHANANDI

NANDYAL DISTRICT-518502, A.P.

Tender Notice No			:  NIT. No:1/ Dr.YSRHU/HRS-MND/2025-26

Date of Commencement of			: 17.09.2025

Downloading of the

bid document

Last date for receipt of bids. 			: 26.09.2025 AT 04.00PM

TIME AND DATE OF

Presentation of  Detailed Project Report	: 27.09.2025 AT 10.30AM

and opening of Technical and Price bid  at

Dr Y S R HORTICULTURAL UNIVERSITY

HORTICULTURAL RESEARCH STATION - 518502

MAHANANDI, NANDYAL (Dt.)

ADDRESS FOR COMMUNICATION:

THE SENIOR SCIENTIST (PL.PATH) & HEAD

HORTICULTURAL RESEARCH STATION MAHANANDI, MC FARM-518502

NANDYAL DISTRICT, ANDHRA PRADESH

Cont. No: 7382633690, 9885522496

Email: hrs-mndi@drysrhu.edu.in

The tender document containing terms and conditions for the execution of this project along with specifications and EMD to be paid are appended.

TERMS AND CONDITIONS

  - The location of the plant is at Horticultural Research Station-MAHANANDI, Nandyal District of Andhra Pradesh.

  - The bidder should quote the rate per square meter (Sqmt) only, including all taxes /VAT/freight/ installation/commissioning/trial/training charges etc in Indian rupees.

  - The required land, buildings, water and power connection will be provided by the Horticultural Research Station and the bidder has to carry out all the works from supply and erection of Structure /equipment and electrical equipment, successful trial run and maintaining the units supplied for 12 months including all maintenance and running cost, duly rectifying all defects during the above period, at the cost of bidder.

  - The bidders are also required to furnish their experience certificate along with details of previously erected plants with that of similar specifications.

  - The erection of structure/supply of equipments must be carried out as per the standard specifications under the supervision of concerned authority of Horticultural Research Station and has to be got certified by him, duly handing over all warranty certificates for the structure and electrical equipments.

      - Supply, erection of all structures/equipments on Turn Key basis. The total project is to be set-up on Turn-key Basis including inter-connecting pipeline / irrigation sections. Bids will be evaluated on Turn-key basis / Section wise basis. All bidders must quote accordingly; otherwise bid will be summarily rejected.

2.  All taxes and duties to be paid to the government needs to be included in price quoted in all the bills.

V)	The bids should be submitted with in time specified at the office of Senior Scientist (Pl.Path) & Head, Horticultural Research Station, Mahanandi along with EMD as specified here under against each item.

| Sl.No | Work/Equipment | EMD Amount to be paid (in Rupees) |
| --- | --- | --- |
| A | Works/ Structures |  |
| 1 | Screen House for sweet orange | 5,00,000-00 |

Bidders must pay the EMD amount specified as above against each of the item in the form of crossed D.D. drawn in favor of The Comptroller,              Dr YSRHU, Venkataramannagudem, Tadepalligudem payable at Union Bank/ any nationalized bank,  Tadepalligudem. Tender documents without earnest money deposit will be rejected and processing fee of Rs.1,000/- (Rupees One thousand only) through DD in favor of The Comptroller, Dr YSRHU, Venkataramannagudem, Tadepalligudem payable at Andhra Bank/ any nationalized bank,  Tadepalligudem which is non-refundable.

VI)   Specifications given are indicative and the tender is to give a detailed description and specifications on each section. The same would be given weightage in finalizing the tender. Rate of the each equipment mentioned should be quoted compulsorily.

VII)     The material/equipments to be supplied by the tenderers should be branded and meet the quality standard and as per laws Norms. Period of warranty shall be 1 year from the date of supply.

VIII)   The specifications and other conditions prescribed in Annexure – A of this

Tender shall also be treated as part of these tender documents for all purposes.

  - Terms of  Payment:

| 1) Upon supply & erection of the total equipment and after successfully trial run of the unit. | 97% of the project cost. |
| --- | --- |
| 2) After maintenance period of one year. | 3% of the project cost. |

ELIGIBILITY CRITERIA:

- The preference would be given to constructor/manufacturer with experience of having under taken complete project on Turn-key Basis and having executed at least five similar projects in reputed Horticultural  or Agricultural Universities / State department of Horticulture / ICAR  institutes/private organizations or similar Government aided University or College in India or Abroad

- The Tenderer should have executed at least 5 similar projects on Turn-key basis in last 5 years of similar or more value on tendered. Certificate from the project client for award of contract and stage of satisfactory project completion (in terms of value) in original or its notarized copy for each project claimed to be submitted.

- The company bidding should be in the industry for at least 10 years in the same filed. Tenderer should have registered in SSI / Directorate of Industries as a Manufacturer or authorized dealer of a reputed company (Please attach certificate).

- Tenderer should have Turnover of at least 5 crores towards construction of Screen Houses exclusively.  Audited financial statements duly certified by chartered accountant for the last five financial years ending August, 31st, 2025.

- The buyer reserves the right to evaluate the tender on Technical Presentation/ capabilities and is not bound to accept lowest tender / quotations.

Evaluation of Bid:

- The Technical Bid Documents will be opened first and evaluated by the Tender Committee. Financial Bid Documents of only those bidders will be opened who have qualified in Technical Bid.

- The Competent Authority reserves the right to accept or reject any tender without any reason thereof.

- Prices to be quoted on FOR basis including all taxes including VAT/freight/ installation/commissioning/trial/training charges etc in Indian rupees.

- It is necessary to submit all the relevant documents like Pan Card, ISO Certification, VAT/Sales Tax Registration, SSI/ Manufacturing Registration, Certificate of Incorporation / Partnership Deed.

- Tenderer should submit the copies of Purchase Orders in support of their genuineness in supplying of the goods in reputed Universities / Colleges / Departments/institutes/private organizations. The tenderer should submit users list for the last 5 years including for term key basis projects involving erection of screen house and equipments.

The Dr YSRHU reserves the right to verify the claims made by the Bidder and to carry out the capacity assessment of the bidder and the Dr YSRHU decision shall be final in this regard.

TECHNICAL SPECIFICATIONS FOR TEND

Construction of  SCREEN HOUSE FOR SWEET ORANGE

- Overall Area: minimum of 1000 Sqmt to maximum of 4000 Sqmt with Side Height 5Mtr x Centre height 6.2Mtr.

*Project dimensions are flexible and will be finalised as per the site conditions.

STRUCTURE:

All Structures, Rafters, Perlins, Trusses are hot dip galvanized and designed as per IS875 standards to withstand wind speed 120 km./hr.

## Frame Works Structure:

Its assembly style without any welding point. All main parts are hot dip galvanized

- Columns: Hot Dip Galvanized Rectangular Profile, tubular Structures.

- Corner Columns: Double Column, Hot Dip Galvanized Rectangular Profiles

- Gutters: Hot Dip Galvanized Steel

- Arches: Hot Dip Galvanized Steel Pipes

- Arch Column Clamp: Galvanized Steel Sheet

- Crop Bar: Galvanized Pipe

- Cross and Reinforcements:

- Corner Reinforcement:                         Hot Dip Galvanized Rectangular Profile

- Column-Arch Reinforcement             Galvanized Pipes

- Column- Column			            Galvanized Pipes

- H –Reinforcement			            Galvanized Rectangular Profile

- X-Reinforcement			            Galvanized Pipes

- Crop Bar-Arch Reinforcement	Galvanized Pipes

Pipe sections for different Structural Member (100% without welding):

- Pillar: Hot galvanized rectangular tubes. Distance between pillars on lateral line: 2m. Distance between pillars on central line: 4m.

- Arc

- Ridge

- Crop Bar.

- Trusses Tie: Tube structural member fitted with zinc plated nuts & bolts. Frame is hot galvanized rectangular tubes.

- Gutter: Hot galvanized steel plate

- Roof beam: Hot galvanized rectangular tubes

- Bolts and Nuts: DIN Steel

Nuts and other metallic parts: Includes all the elements required for joining and water tightens components (such as fittings, clamps, screws and nuts plated against corrosion).

BUFFER ROOM:

- Ante Room: Covered with polycarbonate sheet and the partion would be provided in partition in the buffer room as per user’s requirement.

- Trap Door: long & wide, normally lockable made with clear 6mm polycarbonate glazing, top & bottom tracks, jambs, flashings & installation hardware

- Pre-Entry Room & Door: One pre-entry room is provided and covered with clear multi wall UV stabilized clear polycarbonate sheet with aluminum Profile, EPDM gasket, Silicon sealant and accessories. This pre entry room consist of 4 entrances, one main entrance, Opposite to main entrance another entrance with ramp for small power tiller entry and another two entrances are for Bay-1 and 2 entry. Two entrances will be provided with hinge doors with double leaf made in plastic / FRP sheets mounted in suitable strong frame, and four internal doors are sliding based doors.

CLADDING:

All side walls and roof of the screen house covered by SS 304 grade, 40 mesh net - Complete Set

DRIP IRRIGATION SYSTEM:

- The system is provided with pressurized drip irrigation system with 2” Filter 120 mesh red, Valve/2”, Fertilizer pump, Air valve/1.5”, Pressure relief valve 2”, Pressure gauge, Head unit assembly.

- Drip supply manifold with Valve/2” Aquanet Valve 2”, Lateral 16mm, Drip Net PC 16mmx30cmx1lph, RPVC 40mm 6Kg/cm2, RPVC 63mm 4Kg/cm2.

ELECTRIC WIRING INSIDE SCREEN HOUSE:

- All wires for lighting would be of 2.5 sq. mm FRLS insulated, copper conductor.

- Lighting would be designed to meet a minimum lux requirement of 200lux in screen house and pre-entry room.

- All switches would be of weather proof type.

- Distribution board with MCB control to be provided for protecting all equipment’s.

CIVIL WORK:

- Foundation wall: For all sides of Screen House along with the double door room. WIDE Based 1ft. below earth’s surface, 2ft. above earth’s surface, as kick-board 9” wide. Frame base block height 2’.with plaster wall.

- Floor : Natural Soil (as it is)

- Foot path : 1m to 1.5m wide and 10cm thick (minimum) foot paths will  be provided in the centre (length wise between each bay front & back interconnected) and 1.2m wide plinth protection around the facility made build 60mm paver block will be provided.

- Drainage System: Provision for proper drainage with slope will be provided.

Miscellaneous Items: Miscellaneous washbasin, aluminium ladder, plumbing polymer pipes etc. All fittings shall be ISI marked.

SELECTION PROCEDURE, MARKING SYSTEM & AWARD OF CONTRACT FOR ESTABLISHMENT OF SCREEN HOUSE

Considering the specific requirement of complex technology involved in the project the contract shall be awarded on quality cum cost-based system (QCBS). For final evaluation 75% weightage will be given to technical proposal & 25% to the financial proposal

EVALUVATION PROCESS & MARKS DISTRIBUTION

|  | Total Marks | Weightage | Qualifying Marks |
| --- | --- | --- | --- |
| Technical Evaluation | 100 | 75% | 75 |
| Financial Evaluation | 100 | 25% | NA |
| Final Evaluation | For Final Evaluation, 70% weightage will be given to technical proposal & 25% to the financial proposal. |  |  |

.

SCORING BASED TENDER EVALUATION CRITERIA TO SELECT MOST SUITABLE BIDDER TO EXECUTE THE WORK.

*NOTE: COMMERICAL BIDS WILL BE OPENED ONLY FOR THE COMPANIES WHO SCORE MINIMUM 75 MARKS

MARKS IN THIS EVALUATION PROCESS

| S. No. | EVALUATION CRITERIA | MAXIMUM SCORE |
| --- | --- | --- |
| 1 | The Bidder/Applicant should have registered with valid ISO Certifications like ISO 45001:2018, ISO 9001:2015 & ISO 14001:2015. | 5 |
| 2 | Evidence of a completion certificate for minimum of five projects of screen houses executed with Government institutes in India, in a land area of minimum 2000 sqmt located within India | 15 |
| 3 | The Bidder/Applicant should have registered with PF & ESIC Govt. of India, Documentary proof should be attached. | 5 |
| 4 | The Bidder/Applicant should have valid DSIR certificate | 5 |
| 5 | Preference will be given to those who have previously established a project at YSR University | 15 |
| 6 | The Applicant must be a legal entity like Private Limited/Public Limited/Government Registered Company registered under the Companies Act, Limited Liability Partnership (LLP) under the Partnership Act, Partnership Firm, Societies Registration Act, etc. The agency/s must have valid PAN card and GST Registration. | 5 |
| 7 | The bidder should provide ITR (Income Tax Return) for last three financial years and shall have Annual turnover for last three financial years not be less than INR 10 Cr. The copies of the Profit & Loss statement also to be provided. The bidder should not be in loss in the last 03 financial years. The Turnover Certificate duly signed & stamped by Chartered Accountant. | 10 |
| 8 | The Bidder Make and Complete Specifications of the Material should be clearly mentioned and catalogue/Leaflet/Brochure if any to be submitted with proposal. | 5 |
| 9 | The Applicant must be an original manufacturer, need to submit valid Company Incorporation Certificates with MOA. | 15 |
| 10 | Evaluation by the Technical Committee based on the detailed technical presentation on the similar project expertise provided by bidder. | 20 |
| TOTAL MAXIMUM MARKS |  | 100 |

*Note: Based on the availability of land, tenderer must be prepared to design screen houses at different locations within the research station.

## Annexure – A

(General Conditions)

### Submission of Tender and Deposit of earnest money

- Quoted rate (per square meter) should be written legibly in ink or type written. No alterations should be made to any of the terms and conditions of the tenders by scoring out, altering or overwriting; similarly no alterations are permitted in the rates quoted by them. No alterations will be allowed after this Office receives the tender. Ambiguity must be avoided in filling the tenders. However, any corrections etc., made will have to be duly attested with dated signatures and official seal. The tenders not complying with these conditions will be rejected summarily.

- The tenders not conforming to the prescribed terms and conditions of the Horticultural University or conditional Tenders or Tenders which cannot adhere to the prescribed time schedule are liable for rejection.

#### Validity of rates and other Conditions

- The defect liability period for the Equipments and erection of screen house shall be given for 12 months or more from the date of installation. In the event of any correction or defects or replacement of defective material done during this period, it should be corrected/ replaced at the cost of the bidder/ agency.

- Bids shall remain valid for a period of 90 days from the date of opening of the bid prescribed. In exceptional circumstances, the University may solicit the bidder’s consent for an extension of the period of validity. The request and response shall be made in writing / cable / telex / fax / e-mail.

- The validity of the tender will extend for a period of three months from the date of placing the initial order and it shall be open to the Dr.YSRHU to place the orders with the suppliers on the same rates, terms and conditions for any additional quantities likely to be recurred during that period.

- The bidders shall submit the technical bid and price bid in separate covers and kept in a single cover.

- The inner and outer envelops shall bear the following address:

THE SENIOR SCIENTIST (PL.PATH) & HEAD

HORTICULTURAL RESEARCH STATION MAHANANDI, MC FARM-518502

NANDYAL DISTRICT, ANDHRA PRADESH

- The inner envelopes should also contain the name and address of the bidder.

- Telex, cable, e-mail or facsimile bids will be rejected.

- Bidding Documents must be received by the University at the address specified not later than the time and date specified in the invitation (Notification) for bids. In the event of the date specified being declared as a holiday for the University, the bids will be received up to the appointed time on the next working day.

- The Bidder’s representatives who are present shall sign in the register evidencing their attendance.

- During evaluation of bids subsequent to opening, the University may at its discretion, to ask the bidder for clarification of its bid. The request for clarification and the response shall be in writing and no change in the bid will be entertained.

- The evaluation of the bid will take into account, the past experience in addition to the bid price. Such price should include all duties and taxes to be paid or payable on components of equipment.

12. The bidder may present the project on power point presentation to    		 	Committee on the following.

- Equipment specification

- Operation and maintenance

13. Successful bidder should also take into account the available equipment at the time of setting up the turn key operation and the cost will be reduced as per the quoted item.

# Acceptance of tenders

- Price bids will be opened after completion of power point presentation in the presence of such tenderers who are qualified in technical bid as may be present. A decision with regard to acceptance of tender will be taken as soon as possible.

- The successful tenderer (s) will be intimated by letter (s) or other means of communication and the tenderer (s) so informed shall be bound from the time of transmission of such acceptance by the University. Formal acceptance of the tender (s) will be forwarded to successful tenderer (s) in due course but it will serve merely as a confirmation of the initial information and shall not affect the time from which the offer is/are is bound by the contract(s).

- The University is not bound to accept the lowest tender. Any or all the quotations may be rejected without assigning any reasons. It reserves the right of acceptance in whole or part of the offer made. The decision of the Dr.YSR Horticultural University in the matter shall be final and binding on the tenderers.

TENDER EVALUATION

The evaluation and comparison of the bids shall be done for the technical as well as financial aspects.

a) Technical Bid Evaluation:

While power point presentation or the examination of the documents submitted by the tenderer, the committee shall have the right to verify the claims of experience made by the bidders, based on the bid evaluation, only technically qualified bidders shall be short listed (power point presentation). The financial bid of only technically qualified Bidders shall be opened.

b) Financial Bid Evaluation:

The total cost of the project (landed cost) quoted by the bidder would be considered for financial bid evaluation.

Committee:

  - The committee shall do the above evaluation. The committee shall determine the approach and methodologies for the issues, which may arise during the above, referred evaluation process and their decision is final. The decision of the committee to reject or accept shall be final and binding on all the bidders.

  - The successful bidder shall be responsible for training of the working personnel of Dr.YSRHU for the operation and maintenance of the entire plant for at least 14 days. Further for a period of one year from the date of commissioning and trial run, the bidder shall deploy at least one experienced person from time to time for advising any doubts regarding operation and maintenance besides troubleshooting when ever required.

  - The bidder may submit separate quotation for comprehensive maintenance contract for one year from the date of expiry of initial one year.

#### Other contractual obligations

- The contract shall not be deferred/ modified except by written consent by both the Dr. YSRHU and the Bidder and the Horticultural University shall not in the absence of the specific written acceptance be bound by any provision of the supplier’s quotations, offers etc., which purport to impose conditions, at variance with this contract.

- The Bidder shall not sublet or delegate this contract or part thereof without the written consent of the Horticultural University. Such consent shall not, however, be withheld unreasonably.

- The Bidder shall keep confidential all matters concerning this contract and comply with all reasonable security requirements. All drawings, blocks, specifications, manuscripts, samples etc., supplied by the Horticultural University and all copies thereof shall be returned to the University when their use is terminated. In no event, the Bidder shall permit publicity concerning this contract without prior consent of the Horticultural University.

- No undertaking or commitment given by or made by any officer of the University  verbally or in writing does not have any validity unless it is signed again by the authority competent who concluded an agreement earlier.

- The agreement is to be concluded by the bidder who has signed the tender document only.

#### Consequence of non-supply and damages

- All risks of loss, damage or depreciation to Equipment shall be upon the supplier until the material is delivered at the addresses specified and in accordance with the provision of the contract. Till the material is received at the respective destination indicated by the university, the property continues to be at the risk of the Bidder. The mere fact that the material is delivered to transporter is no defense to the Bidder and the Bidder will be squarely held responsible for any delayed receipt of the material by the University or for loss or damage of any kind to the material in transit.

- Assuming that the bidder fails to deliver any or all the material covered by the contract, the Horticultural University reserves the right in addition to other legal remedies, to cancel the contract or any portion thereof and hold the Bidder liable for all damages sustained by the university by virtue of the Bidder failing to perform the contract and consequent cancellation of the contract.

- In the event of the supplier (Bidder) failing to complete the supply in time or according to the approved specifications, the university reserve the right to make such arrangements as it may think fit for the completion of the supplies on account of and at the sole risk of the bidder.

- The time allowed for delivery of goods shall be deemed to be the essence of the contract. The University also reserves the right to cancel the purchase order in case supplies are delayed beyond the scheduled date of delivery and to make such arrangements as it may think fit for the completion of supplies on account and at the risk of the suppliers (s). The additional expenses thus incurred together with the consequential losses and also the liquidated damages shall be recovered from the supplier out of his / their security deposit / earnest money deposit and any other amount due to him / them. The balance still, if any, payable by the supplier shall be paid by him/them within 7 days of notice by the Dr.YSR Horticultural University

FORFEITURE/REFUND OF THE EARNEST MONEY DEPOSIT (EMD)

- In case the selected Tenderer (s) does not supply the equipment at the quoted rates within the period of contract and commits any breach of any one or more of these terms and conditions, the Earnest Money Deposit deposited by the Tenderer (s) will be forfeited.

- Earnest Money of the unsuccessful Tenderer (s) shall be refunded within three months from the date of decision regarding the tenders. No interest is payable by the University on such deposit.

- On due performance and satisfactory completion of the order in all respects during the contract periods, the Earnest Money Deposit (Security Deposit) will be refunded to the Contractor(s) without interest within a period of 3 months with effect from the date receipt of a request to this effect from the Bidder .

#### SETTLEMENT OF DISPUTES

#### Any difference or dispute arising out of or in connection with this tender or acceptance thereof or the contract that may be entered in consequence thereof, shall be decided by arbitration. The Chairman of the committee for purchase of the equipment, Dr YSR Horticultural University or his nominee shall be the sole arbitrator and the arbitrator’s decision shall be final and binding on the parties. The Tenderer (s) will have no objection to such appointment on any ground whatsoever including that such nominee, in his official capacity dealt with this matter at any stage.

- The parties hereby agree that in the event of any dispute no cause of action shall arise in their favor to approach any court unless they have restored to and exhausted the remedy of arbitration as envisaged above.

- The parties also do hereby agree that the contract envisaged by these terms and conditions shall be deemed to have been entered into at Hyderabad and the courts at Hyderabad alone will have jurisdiction to try and legal proceedings which may arise out of this contract. Neither party shall file any proceedings in any other Court.

TENDER FORM

From :					To

The Senior Scientist (Pl.Path) & Head,

Horticultural Research Station,

Dr YSR Horticultural University

Mahanandi - 518502. (A.P)

.

Ref:  1.Your tender Notice NIT No:1/Dr YSRHU/HRS-MND/2025-26   Dated...........

2. EMD – D.D. No. ...................Dated..........................for Rs ............................

I/We have read the contents of the terms and conditions mentioned in your tender schedule and its enclosures and agree to abide by the same.

I/We hereby offer to supply the articles prescribed in the schedule (or such portion thereof as you may specify on the acceptance of tender) at the price given below.

I/We agree to hold this offer open for a period of 90 days from the date of acceptance of tender and shall be bound by communication of acceptance dispatched within a period of 15 days from the date of accepting the tender of the bidder.

I/We have also examined the requisite specifications of the material to be supplied and my / our offer is to supply the required material in accordance with the requisite specifications.

I/We have carefully considered all terms and conditions in Annexure A and B and particulars regarding settlement of disputes and we have signed the same in token of consciously accepting the same and do hereby state that we accept them without any reservations and accordingly I/We quote the rates inclusive of all taxes, duties, transportation, insurance etc., as below.

Name of the item                                                             Unit price/ per square meter

The articles will be ready for delivery within ..........................days from the date of receipt of firm orders.

Yours faithfully,

Proprietor

(Signature and stamp of the Tenderers

State legal status, whether Prop./

Partner / Registered firm / Company etc.)

Encl: Samples enclosed

EMD-DD No.......................................dated............................for Rs...........................

Dr.Y.S.R. HORTICULTURAL UNIVERSITY

Dr.YSRHU - HORTICULTURAL RESEARCH STATION - MAHANANDI

Phone: 73826 33690, 9885522496,  E-mail: hrs-mndi@drysrhu.edu.in

TENDER NOTICE

NIT No:1/Dr YSRHU/HRS-MND/2025-26                             Dated:

Bids are invited from firms towards construction of Screen house. For details visit the www.drysrhu.edu.in.

SENIOR SCIENTIST (PL.PATH) &HEAD